Shortest-Path Planning in a Large-Scale IoV/UAV Mobility Network

DAA Laboratory Project | Comparative Algorithmic Performance Evaluation
Benchmark: 9th DIMACS USA Road Network (New York: USA-road-d.NY) | Date: October 2026

# 1. Executive Summary

This project presents a rigorous computational study and empirical analysis of shortest-path routing algorithms applied to realistic, large-scale vehicular mobility (Internet of Vehicles - IoV) networks. Using the 9th DIMACS Implementation Challenge New York road network (264,346 vertices, 733,846 directed edges), we construct a weighted spatial graph and evaluate Dijkstra's Algorithm (implemented from first principles with a binary min-heap) against the A* Search Algorithm (with an admissible and consistent scaled Haversine distance heuristic) across multiple network scales (1,000 to 100,000 vertices).

Key Findings:
• Cost Equivalence: A* and Dijkstra yielded identical shortest-path costs across 100% of evaluated queries.
• Search Space Reduction: A* pruned the search space dramatically, exploring 75.6% to 88.5% fewer vertices and examining 74.5% to 88.1% fewer edges than Dijkstra.
• Execution Latency: A* achieved an average speedup of 2.1x to 5.1x over Dijkstra across all network sizes.
• Dynamic Rerouting (Bonus): Dynamic edge cost penalties modeling traffic incidents (w'(u,v) = α·d + β·c) triggered optimal rerouting, saving 79.85% in path traversal cost.

# 2. Part A: Dataset Selection & Specification

To satisfy the requirement for a realistic large-scale mobility network, we selected the official 9th DIMACS Implementation Challenge USA Road Network (New York State benchmark: USA-road-d.NY). This dataset is derived from US Census Bureau TIGER/Line geographic files.

| Dataset Name | 9th DIMACS Implementation Challenge: USA Road Networks (New York - Distance Metric) |
| --- | --- |
| Source URL | http://www.diag.uniroma1.it/challenge9/download.shtml |
| Primary Citation | C. Demetrescu, A. V. Goldberg, D. S. Johnson (eds.), The Shortest Path Problem, DIMACS Series, Vol. 74, AMS, 2009 |
| Scale / Size | 264,346 Vertices ¦ 733,846 Directed Arcs (Road Segments) |
| Graph Modeling | Vertex = Road intersection/waypoint; Edge = Directed road segment; Weight = Road distance (decimeters) |
| Suitability Rationale | Real urban road geometry, authentic one-way restrictions, exact coordinates for A*, standardized scientific benchmark |

# 3. Part B: Graph Construction & Topological Analysis

Preprocessing steps applied:
1. Vertex Relabeling: Converted 1-based indices to 0-based integers.
2. Degeneracy Filtering: Discarded self-loops and non-positive weights.
3. Coordinate Transformation: Scaled micro-degrees (10^-6 deg) to decimal degrees and precomputed radians.
4. Adjacency List: Built adj[u] = [(v, w), ...] storing only required neighbors, ensuring low memory footprint (~45 MB).
5. Subgraph Generation: BFS outward from a central high-degree node to generate realistic, topologically connected, geographically compact subgraphs.

| Graph Scale | Vertices (¦V¦) | Edges (¦E¦) | Avg Degree (E/V) | Max Degree |
| --- | --- | --- | --- | --- |
| 1,000 | 1,000 | 3,468 | 3.4680 | 8 |
| 5,000 | 5,000 | 16,800 | 3.3600 | 8 |
| 10,000 | 10,000 | 33,992 | 3.3992 | 8 |
| 25,000 | 25,000 | 84,532 | 3.3813 | 8 |
| 50,000 | 50,000 | 161,546 | 3.2309 | 8 |
| 100,000 | 100,000 | 304,170 | 3.0417 | 8 |

# 4. Parts C & D: Algorithms & Mathematical Complexity

Dijkstra's Algorithm Implementation:
• Implemented with a binary min-heap (heapq) using lazy deletion to handle distance relaxations in O(log V) time.
• Early stopping triggers as soon as destination D is popped from the heap.
• Theoretical time complexity: O((V + E) log V). For sparse road networks (E ≈ 3V), practical runtime is O(V log V).
• Space complexity: O(V + E) for adjacency list, parent array, distance array, and priority queue.

A* Search Algorithm Implementation:
• Evaluates priority f(v) = g(v) + h(v), where h(v) = scale · haversine(v, D).
• The scaling constant scale = min_{(u,v)} [weight(u,v) / haversine(u,v)] = 7.152478 guarantees that for every edge, weight(u,v) ≥ scale · haversine(u,v).
• Admissibility: Straight-line spherical distance is the infimum of any physical path. Multiplying by scale ensures h(v) ≤ d*(v, D).
• Consistency (Monotonicity): By the triangle inequality on the sphere, haversine(u, D) ≤ haversine(u, v) + haversine(v, D). Scaling yields h(u) - h(v) ≤ scale · haversine(u, v) ≤ weight(u, v), proving consistency.
• Consistency guarantees that each node is settled at most once without needing re-opening.

# 5. Part E: Experimental Evaluation & Results

Benchmark queries were evaluated across 6 network scales using 10 reachable (S, D) pairs per scale with 3 timing repeats:

| Graph Size | Algorithm | Avg Shortest Cost | Vertices Explored | Edges Examined | Time (ms) |
| --- | --- | --- | --- | --- | --- |
| 1,000 | Dijkstra | 13,592.90 | 389.5 | 1,398.2 | 0.211 |
| 1,000 | A* | 13,592.90 | 44.8 (-88.5%) | 166.0 (-88.1%) | 0.079 (2.67x) |
| 5,000 | Dijkstra | 54,194.20 | 2,918.8 | 9,895.2 | 4.492 |
| 5,000 | A* | 54,194.20 | 633.6 (-78.3%) | 2,203.3 (-77.7%) | 1.795 (2.50x) |
| 10,000 | Dijkstra | 46,282.10 | 3,010.8 | 10,313.2 | 4.376 |
| 10,000 | A* | 46,282.10 | 441.5 (-85.3%) | 1,519.3 (-85.3%) | 0.854 (5.12x) |
| 25,000 | Dijkstra | 38,256.70 | 2,019.2 | 6,878.0 | 2.415 |
| 25,000 | A* | 38,256.70 | 490.4 (-75.7%) | 1,676.0 (-75.6%) | 1.136 (2.13x) |
| 50,000 | Dijkstra | 46,910.20 | 2,203.2 | 7,540.2 | 2.978 |
| 50,000 | A* | 46,910.20 | 482.4 (-78.1%) | 1,635.3 (-78.3%) | 1.097 (2.71x) |
| 100,000 | Dijkstra | 65,590.00 | 3,398.6 | 10,521.7 | 4.542 |
| 100,000 | A* | 65,590.00 | 828.6 (-75.6%) | 2,522.8 (-76.0%) | 1.771 (2.56x) |

# 6. Performance Plots & Visual Analysis

Figure 1: Comprehensive Comparative Dashboard (Execution Time, Vertices Explored, Edges Examined)

# 7. Part F: Analytical Answers to Core Questions

Q1. Suitability of Selected Dataset
The 9th DIMACS USA Road Network (New York) is uniquely suited because it models authentic vehicular road infrastructure derived from US Census TIGER/Line data. It contains 264,346 nodes, realistic one-way constraints, physical distance weights, and exact geographic coordinates for A* heuristics. It provides a standardized, cited, and reproducible benchmark.

Q2. Transformation of Raw Data into Graph
Raw .gr and .co files were streamed line-by-line. 1-based indices were shifted to 0-based. Self-loops and non-positive weights were filtered out. Micro-degrees were converted to decimal degrees and radians. To evaluate subgraphs, a BFS was performed from a central node to extract compact, topologically connected metropolitan regions rather than shattered random nodes.

Q3. Cost Equivalence between Dijkstra and A*
Dijkstra and A* produced identical path costs across 100% of tested queries. Because our scaled Haversine heuristic is admissible and consistent (h(v) <= d*(v, D)), A* is mathematically guaranteed to return the optimal shortest path.

Q4. Search State Exploration Comparison
A* explored 75.6% to 88.5% fewer vertices than Dijkstra. Dijkstra is an uninformed search that expands radially outward in all directions (circular topological wave). A* incorporates h(v) into f(v) = g(v) + h(v), directing exploration into a narrow elliptical corridor pointing directly toward the destination.

Q5. Scaling of Execution Time vs. Graph Size
Execution time scales sub-linearly with graph size when queries span regional boundaries, and near-linearly with path hop length. With early termination, search time is governed by the radius d*(S, D) rather than total graph size. A* maintained an execution time of 0.08 ms to 1.77 ms, consistently 2x to 5x faster than Dijkstra.

Q6. Impact of Graph Density
Road networks are sparse spatial planar graphs (avg degree ≈ 3.0, max degree 8). Sparsity ensures each settled node explores only 2-4 edges, keeping binary heap operations cheap (O(log V)). In dense graphs (E ≈ V^2), heap pushes explode to O(V^2), degrading binary-heap Dijkstra to O(V^2 log V) and diminishing heuristic corridor pruning.

Q7. Theoretical Complexity of Dijkstra
Our Dijkstra uses an adjacency list and binary min-heap with lazy deletion: V pop operations and at most E push operations, each taking O(log V) time. Total time complexity is O((V + E) log V). For sparse networks where E = O(V), practical runtime is O(V log V). Space complexity is O(V + E).

Q8. Heuristic Justification, Admissibility, and Consistency
We used scaled Haversine distance: h(v) = scale · haversine(v, D). By setting scale = min_{(u,v)} [weight(u,v) / haversine(u,v)], we guarantee weight(u,v) >= scale · haversine(u,v) for every edge. By the triangle inequality on the sphere, h(u) - h(v) <= scale · haversine(u, v) <= weight(u, v), proving both consistency and admissibility.

Q9. Conversion Challenges
Key challenges included: coordinate micro-degree units, decimeter distance units requiring heuristic scaling, streaming large compressed files efficiently, and extracting connected subgraphs using BFS rather than random sampling.

Q10. Architectural Recommendation for IoV/UAV Route Planning
We strongly recommend A* with a consistent geographic heuristic for point-to-point IoV and UAV route planning. It provides provably optimal paths while saving 75-88% in CPU expansions and cutting query latency by 2x-5x, essential for high-throughput edge navigation servers. Dijkstra remains suitable when no coordinates exist or for single-source multi-destination queries.

# 8. Bonus Task: Dynamic Edge Costs & Congestion Rerouting

Dynamic edge costs were modeled as w'(u,v) = α·d(u,v) + β·c(u,v). Under a simulated traffic incident on 40% of the baseline route (c = 5d) and ambient delay (c = 1..3d):
• Baseline Route under Incident: 197,065.19
• Dynamic Rerouted Optimal Route: 39,700.00 (Cost Saving: 157,365.19 or 79.85%)
• Overlap with Original Route: 6 / 23 nodes (26.1%)
• Algorithmic Performance: A* explored only 244 nodes (vs. 2,440 in Dijkstra) in 1.35 ms, finding the exact optimal detour.